# Ergonomic Bench Assembly & Securing Procedure

Technical Installation Manual — Calf Panel, Seat Panel Support & Rear Retention Block

Fasteners: M6 x 30 mm wood screws, Phillips head. Driver clutch stage 2–3 (approx. 1.5–2.0 N·m) — do not exceed 2.5 N·m. Disconnect any wiring near the seat panel support before starting.

# Step 1: Fix the Calf Panel to the Seat Panel Support

- Seat the calf panel's lower edge fully into the seat panel support frame, flush with no gap.

- Align the cross brace's pilot hole over the calf panel hole (see Figure 1).

- Drive one screw through the brace into the panel until flush — do not countersink below the surface.

- Check rigidity: no lateral movement or creak when the panel is pushed.

Figure 1. Calf panel secured to the seat panel support.

# Step 2: Fix the Wooden Block to the Rear of the Seat Panel

- With the seat panel face-down, position the block flush against the rear edge, matching its placement in Figure 2. The block already has pilot holes drilled through it — the seat panel does not.

- Holding the block in place, use its pilot holes as a guide to mark the drilling points on the seat panel, then drill a matching 3 mm pilot hole at each mark.

- Hand-start both screws through the block into the pilot holes before driving either fully, keeping the block flush and square against the edge.

- Drive both screws evenly, alternating to keep the block square. Do not over-tighten — excess torque can split the grain.

Figure 2. Block with pilot holes — use it to mark drill points on the panel.

# Step 3: Install the Seat and Anchor to the Rear Wall

- Lower the seat assembly into the enclosure, guiding the supports into place.

- Slide it rearward until the wooden block contacts the rear wall; confirm the seat sits level.

- Drive one screw at each of the two marked points where the block meets the rear wall (Figure 3).

- Recheck Step 1 and 2 joints, then load-test: press firmly at front, center, and rear — no deflection or creak.

Figure 3. Completed installation — block fastened to the rear wall.

CAUTION: Do not sit on the bench until all fastening points above are installed and the load test has passed.